LAB 5_Web Server (Code)

| Name : | Tran Le Minh Quan |
| --- | --- |
| Your student ID : | 22CE.B022 |

- main.py in Send ESP32:

  - Set up Network and ESP-NOW:

wlan = network.WLAN(network.STA_IF)

wlan.active(True)

e = espnow.ESPNow()

e.active(True)

  - Set up LED and DHT11 pin:

led = Pin(2, Pin.OUT)

sensor = dht.DHT11(Pin(32))

def blink(duration = 500):

led.value(1)

time.sleep_ms(duration)

led.value(0)

  - Add MAC address of Receive ESP32:

SLAVE_MAC_ADDRESS = b'\x14\x2B\x2F\xC5\xD8\x20'

try:

e.add_peer(SLAVE_MAC_ADDRESS)

print(f'Added peer: {SLAVE_MAC_ADDRESS.hex('-')}')

except OSError as er:

print(f'Error: {er}')

  - Read data from DHT11 and package it:

try:

sensor.measure()

temp = sensor.temperature()

hum = sensor.humidity()

print(f'Readed: temp<{temp}> - hum<{hum}>')

except OSError as er:

print('Error when read from DHT11!!')

temp = 0

hum = 0

packet_id += 1

timestamp = time.ticks_ms()

data_send_json = {

'id': packet_id,

'ts': timestamp,

'temp': temp,

'hum': hum

}

data_send = json.dumps(data_send_json)

  - Send data and wait to Receive ESP32 reply:

data_bytes = data_send.encode()

try:

e.send(SLAVE_MAC_ADDRESS, data_bytes)

print(f'Sended: {data_send}')

blink()

except:

print('Error when send data!!!')

time.sleep(3)

continue

host, msg = e.recv(2000)

if msg:

blink()

reply_ts = time.ticks_ms()

delay = time.ticks_diff(reply_ts, timestamp)

print(f' -> Reply from <{host.hex('-')}>: {msg.decode()}')

print(f' -> Delay time: {delay}ms')

elif host is None:

print('No reply')

- main.py in Receive ESP32:

2.1. Set up Network and ESP-NOW and Set up LCD, LED pin:

#SET UP LCD & LED

I2C_ADDRESS = 0x27

SCL_PIN = Pin(21)

SDA_PIN = Pin(22)

i2c = I2C(0, scl = SCL_PIN, sda = SDA_PIN, freq = 400000)

lcd = I2C_LCD(i2c, I2C_ADDRESS)

led = Pin(2, Pin.OUT)

led.value(0)

def blink(duration = 500):

led.value(1)

time.sleep_ms(duration)

led.value(0)

wlan = network.WLAN(network.STA_IF)

wlan.active(True)

e = espnow.ESPNow()

e.active(True)

2.2. Wait to receive data, process it and send reply to Send ESP32:

while True:

host, msg = e.recv()

if msg:

receive_ts = time.ticks_ms()

blink()

data_received = msg.decode()

print(f'Received data')

if host not in peer_list:

try:

e.add_peer(host)

peer_list.add(host)

print(f'Added: {host.hex('-')}')

except OSError as er:

print('Error when add new peer: {er}')

try:

data_json = json.loads(data_received)

send_time = data_json.get('ts')

delay_fromSend = time.ticks_diff(receive_ts, send_time)

lcd.display_str(f'Id:{data_json.get('id')}, D:{delay_fromSend}ms     ', 0,0)

lcd.display_str(f'T:{data_json.get('temp')}C, H:{data_json.get('hum')}%', 0,1)

print(f'- JSON DETAIL FROM {host.hex('-')}: {msg.decode()}')

print(f'=> id: {data_json.get('id')}')

print(f'=> temp: {data_json.get('temp')}')

print(f'=> hum: {data_json.get('hum')}')

print(f'=> Delay from send time: {delay_fromSend}ms')

reply = b'Ack_OK'

except OSError as er:

print(f'Error when translate Json: {er}')

reply = b'Ack_ERROR'

try:

e.send(host, reply)

print(f'Reply <{host.hex('-')}>: {reply.decode()}')

blink()

except OSError as er:

print(f'Error when reply: {er}')

- Distance effect:

- No obstructions: best as under 50 meter, there will be flickering if the device out of that range

- Have obstruction: best as under 10 meter