LAB 5_Web Server (Code)

| Name : | Tran Le Minh Quan |
| --- | --- |
| Your student ID : | 22CE.B022 |

- boot.py

try:

import usocket as socket

except:

import socket

from machine import Pin

import network

import esp

esp.osdebug(None)

import gc

gc.collect()

ssid = '2uaan'

password = '22222888'

station = network.WLAN(network.STA_IF)

station.active(True)

station.connect(ssid, password)

while station.isconnected() == False:

pass

print('Connection successful')

print(station.ifconfig())

led = Pin(2, Pin.OUT)

- main.py

def web_page():

if led.value() == 1:

led_state = 'checked'

else:

led_state = ""

html = """<html><head><meta name="viewport" content="width=device-width, initial-scale=1"><style>

body{max-width: 400px; margin: 0px auto;}

.switch{position:relative;display:inline-block;width:120px;height:68px}.switch input{display:none}

.slider{position:absolute;top:0;left:0;right:0;bottom:0;background-color:#ccc;border-radius:34px}

.slider:before{position:absolute;content:"";height:52px;width:52px;left:8px;bottom:8px;background-color:#fff;-webkit-transition:.4s;transition:.4s;border-radius:68px}

input:checked+.slider{background-color:#9c5317}

input:checked+.slider:before{-webkit-transform:translateX(52px);-ms-transform:translateX(52px);transform:translateX(52px)}

</style><script>function toggleCheckbox(element) { var xhr = new XMLHttpRequest(); if(element.checked){ xhr.open("GET", "/?led=on", true); }

else { xhr.open("GET", "/?led=off", true); } xhr.send(); }</script></head><body>

<h1>ESP Web Server</h1><h2>Tran Le Minh Quan - 22CE.B022</h2><label class="switch"><input type="checkbox" onchange="toggleCheckbox(this)" %s><span class="slider">

</span></label></body></html>""" % (led_state)

return html

s = socket.socket(socket.AF_INET, socket.SOCK_STREAM)

s.bind(('', 80))

s.listen(5)

while True:

conn, addr = s.accept()

print('Got a connection from %s' % str(addr))

request = conn.recv(1024)

request = str(request)

print('Content = %s' % request)

led_on = request.find('/?led=on')

led_off = request.find('/?led=off')

if led_on == 6:

print('LED ON')

led.value(1)

if led_off == 6:

print('LED OFF')

led.value(0)

response = web_page()

conn.send('HTTP/1.1 200 OK\n')

conn.send('Content-Type: text/html\n')

conn.send('Connection: close\n\n')

conn.sendall(response)

conn.close()