LAB_ESP_NOW(tt)

Tên nhóm: Name

Lớp: 22ES

| Mã sinh viên | Thành viên |
| --- | --- |
| 22CE.B022 | Trần Lê Minh Quân |
| 22CE.B015 | Đinh Hoàng Long |
| 22CE.B025 | Nguyễn Văn Thành Tâm |
| 22CE.B002 | Phạm Vũ Ngọc Dương |
| 22CE.B036 | Phan Nguyễn Tấn Tuyên |

Sender 1

# main.py - MASTER (SENDER)

import network

import espnow

import time

import json

from machine import Pin, I2C, Timer

import dht

# Set up LED and DHT11 pin

led = Pin(32, Pin.OUT)

led_state = 0

led.value(led_state)

led_onboard = Pin(2, Pin.OUT)

sensor = dht.DHT11(Pin(23))

led.value(0)

ID_NODE = 1

def blink(duration = 500):

led_onboard.value(1)

time.sleep_ms(duration)

led_onboard.value(0)

# Set up network and ESP-NOW

wlan = network.WLAN(network.STA_IF)

wlan.active(True)

e = espnow.ESPNow()

e.active(True)

# MAC address of Receiver

SLAVE_MAC_ADDRESS = b'\x14\x2B\x2F\xC5\xD8\x20'

try:

e.add_peer(SLAVE_MAC_ADDRESS)

print(f'Added peer: {SLAVE_MAC_ADDRESS.hex('-')}')

except OSError as er:

print(f'Error: {er}')

def wait_for_toggle(timer):

global led_state

host, msg = e.recv(10)

if msg:

blink()

print(f'Received: {msg.decode()}')

led_state = 1 - led_state

led.value(led_state)

wait_toggle = Timer(0)

wait_toggle.init(period = 100, mode = Timer.PERIODIC, callback= wait_for_toggle)

while True:

# Read data from DHT11

try:

sensor.measure()

temp = sensor.temperature()

hum = sensor.humidity()

print(f'Readed: temp<{temp}> - hum<{hum}>')

except OSError as er:

print('Error when read from DHT11!!')

temp = 0

hum = 0

timestamp = time.ticks_ms()

data_send_json = {

'id': ID_NODE,

'ts': timestamp,

'temp': temp,

'hum': hum

}

data_send = json.dumps(data_send_json) # Turn to json string

data_bytes = data_send.encode()

# Send data to SLAVE_MAC_ADDRESS

try:

e.send(SLAVE_MAC_ADDRESS, data_bytes)

print(f'Sended: {data_send}')

blink()

except:

print('Error when send data!!!')

time.sleep(3)

continue

print('-------------------------------------')

time.sleep(2)

Sender 2

# main.py - MASTER (SENDER)

import network

import espnow

import time

import json

from machine import Pin, I2C, Timer

import dht

# Set up LED and DHT11 pin

led = Pin(32, Pin.OUT)

led_state = 0

led.value(led_state)

led_onboard = Pin(2, Pin.OUT)

sensor = dht.DHT11(Pin(23))

led.value(0)

ID_NODE = 2

def blink(duration = 500):

led_onboard.value(1)

time.sleep_ms(duration)

led_onboard.value(0)

# Set up network and ESP-NOW

wlan = network.WLAN(network.STA_IF)

wlan.active(True)

e = espnow.ESPNow()

e.active(True)

# MAC address of Receiver

SLAVE_MAC_ADDRESS = b'\x14\x2B\x2F\xC5\xD8\x20'

try:

e.add_peer(SLAVE_MAC_ADDRESS)

print(f'Added peer: {SLAVE_MAC_ADDRESS.hex('-')}')

except OSError as er:

print(f'Error: {er}')

def wait_for_toggle(timer):

global led_state

host, msg = e.recv(10)

if msg:

blink()

print(f'Received: {msg.decode()}')

led_state = 1 - led_state

led.value(led_state)

wait_toggle = Timer(0)

wait_toggle.init(period = 100, mode = Timer.PERIODIC, callback= wait_for_toggle)

while True:

# Read data from DHT11

try:

sensor.measure()

temp = sensor.temperature()

hum = sensor.humidity()

print(f'Readed: temp<{temp}> - hum<{hum}>')

except OSError as er:

print('Error when read from DHT11!!')

temp = 0

hum = 0

timestamp = time.ticks_ms()

data_send_json = {

'id': ID_NODE,

'ts': timestamp,

'temp': temp,

'hum': hum

}

data_send = json.dumps(data_send_json) # Turn to json string

data_bytes = data_send.encode()

# Send data to SLAVE_MAC_ADDRESS

try:

e.send(SLAVE_MAC_ADDRESS, data_bytes)

print(f'Sended: {data_send}')

blink()

except:

print('Error when send data!!!')

time.sleep(3)

continue

print('-------------------------------------')

time.sleep(2)

Receiver

# main.py - SLAVE (RECEIVER)

import network

import espnow

import time

import json

import machine

from machine import Pin, I2C

from lcd_api import I2C_LCD

#Set up BUTTON

button1 = Pin(32, Pin.IN, Pin.PULL_DOWN)

button2 = Pin(33, Pin.IN, Pin.PULL_DOWN)

flag_button1 = False

flag_button2 = False

last_press1 = 0

last_press2 = 0

#SET UP LCD & LED

I2C_ADDRESS = 0x27

SCL_PIN = Pin(21)

SDA_PIN = Pin(22)

i2c = I2C(0, scl = SCL_PIN, sda = SDA_PIN, freq = 400000)

lcd = I2C_LCD(i2c, I2C_ADDRESS)

lcd.display_str('----', 6,0)

lcd.display_str('----', 6,1)

led = Pin(2, Pin.OUT)

led.value(0)

NODE1_MAC_ADDRESS = b'\x88\x57\x21\x69\xf0\x20'

NODE2_MAC_ADDRESS = b'\x5C\x01\x3B\x4B\x41\xCC'

def blink(duration = 500):

led.value(1)

time.sleep_ms(duration)

led.value(0)

#Set up network and ESP-NOW

wlan = network.WLAN(network.STA_IF)

wlan.active(True)

e = espnow.ESPNow()

e.active(True)

try:

e.add_peer(NODE1_MAC_ADDRESS)

e.add_peer(NODE2_MAC_ADDRESS)

except:

print('Error when add MAC address!!!')

#Interrupt for toggle LED

def led1(pin):

global last_press1, flag_button1

now = time.ticks_ms()

if time.ticks_diff(now, last_press1) > 200:

flag_button1 = True

button1.irq(trigger= Pin.IRQ_FALLING, handler= led1)

def led2(pin):

global last_press2, flag_button2

now = time.ticks_ms()

if time.ticks_diff(now, last_press1) > 200:

flag_button2 = True

button2.irq(trigger= Pin.IRQ_FALLING, handler= led2)

while True:

host, msg = e.recv(10) # Wait for receive

if msg:

receive_ts = time.ticks_ms()

blink()

data = msg.decode()

data_json = json.loads(data)

delay_time = receive_ts - data_json.get('ts')

if host == NODE1_MAC_ADDRESS:

lcd.display_str(f'N1 T:{data_json.get('temp')}C H:{data_json.get('hum')}% ', 0, 0)

print(f'Received from node 1: {data}')

print(f'Delay: {delay_time}')

if host == NODE2_MAC_ADDRESS:

lcd.display_str(f'N2 T:{data_json.get('temp')}C H:{data_json.get('hum')}% ', 0, 1)

print(f'Received from node 2: {data}')

print(f'Delay: {delay_time}')

irq_state = machine.disable_irq()

if flag_button1:

flag_button1 = False # Hạ cờ ngay lập tức

machine.enable_irq(irq_state)

print("-> Phat hien Nut 1! Dang gui lenh cho Sender 1...")

try:

e.send(NODE1_MAC_ADDRESS, b'change_LED_state')

except OSError as e:

print(f"Loi gui lenh cho S1: {e}")

else:

machine.enable_irq(irq_state)

irq_state = machine.disable_irq()

if flag_button2:

flag_button2 = False # Hạ cờ ngay lập tức

machine.enable_irq(irq_state)

print("-> Phat hien Nut 2! Dang gui lenh cho Sender 2...")

try:

e.send(NODE2_MAC_ADDRESS, b'change_LED_state')

except OSError as e:

print(f"Loi gui lenh cho S2: {e}")

else:

machine.enable_irq(irq_state)